🧠 8086 Microprocessor: Simple Review

Here’s a quick and clear breakdown of the 8086 microprocessor for your revision:

🔹 Basic Specs

- Word size: 16-bit processor

- Address lines: 20 → can access 1MB of memory

- Data lines: 16 → transfers 16 bits at a time

- Clock speed: Typically 5–10 MHz

🔹 Architecture Highlights

- CISC design: Complex Instruction Set Computer — supports rich instructions like multiplication/division

- Segmented memory: Uses segment + offset to access memory (Code, Data, Stack, Extra segments)

- Instruction queue: 6-byte prefetch queue for faster execution (pipelining)

🔹 Operating Modes

- Minimum Mode: Single processor system

- Maximum Mode: Multiprocessor system (uses control signals for coordination)

🔹 Registers

- General-purpose: AX, BX, CX, DX (each split into high/low: AH/AL, etc.)

- Segment registers: CS, DS, SS, ES

- Pointer/index: SP, BP, SI, DI

- Flag register: Contains status flags like Zero, Carry, Sign, Overflow

🔹 Instruction Set

- Supports arithmetic, logical, control transfer, string manipulation, and machine control instructions.

- Example: MOV, ADD, SUB, MUL, DIV, JMP, CALL, RET

🔹 Applications

- Used in early PCs, embedded systems, and as a teaching model for microprocessor architecture.